The three boiling tank brew house for beer making by

dekoktion and infusion system,instaled at one construction.

Typ: ORV-50-Hi3

Used materials:

- Staniless steel no. 17 241 (din:1,4301)

- Copperclad polished

- Yelow brass (decorative comp.)

- Polieuretan for isolation

- Glas ( window in boiler, etc)

# Heating:

# Steam generator

# Used steam:45 kg/h

- Steam output: 7,2 kW

- Mash-pan realy: 76 L

# Mash-pan at work : 50 L

- Large of all two kettles (mash and young beer pan): 500

- High: 1300 mm

- High without chimney: 782

- Regulation of temperature: digital/automatic

- Beer transfer: Hand regulator type GG staniless steel

- Max. input of el. energy : 11,8 kW

- Tension: 3x240/400 V 50 HZ

- Capacity of stirrer: 450 Nm

- Speed of stirrer: regulated 2-20 /min

- Service of transfer: manual

Time for cooking cca 9 Hour +- 1 Hour

Boil kettles contents:

- Steam heating is situated in downer part of kettle:

- capacity of water - 16 L

- Max. pression 3,5 Bar

- Max. permiss. pression 5 Bar

- Electronic regulation of heating 5-120 °C ,range 0,1 °C

- Let out and let in arm. Dn. 20

- input 9 kW

- Heating of dripping water

- Capacity 30 L

- Max. high temperature 95°C

- Cooler of malt

- Material class 17 241 allwelded.

- Max. pression 8 bar

- Transfer sheet 1,85 m2

- Dimension of let in and let out cooling medium DN 25

- Dimension of let in and let out cooling water DN 25

- Transfer pump :

- Type: Calpeda

- Input: 240 V 50 HZ

- Speed: 1600/min

- H-max: 24 m 18 L/min

- Max. temperature 145°C

- Max. density 60%

- Pipeline for malt

- DN 30 mat. 17 241

- DN 20 mat 	17 241

- DN 15 mat 	17 241

- Pipeline for water

- DN 15 mat. 17 241

- Electroregulation IP 55

Measuring and regulation:

- 1x Head switch 3x220V 50Hz 32 A

- 2x Digital termostat Dixel. fx 110 with dimension of regulated temperat. –30+150 °C

- 3x contactor 220V 50 HZ 16A

- 3x switch din 3x 220 V 6A heat.

- 2x Frekvence changer Hitachi LGS 100 with speed regulation 0-60 HZ

- 4x contactor 220 V 50 Hz 6a

- 4x switch 220 V 50 Hz

- 8x light controll 3x red light for boiling and 4 x green for motors

- 2x secure 220 V 50 Hz 10 A

- 2X switch 220 V

- 2x Speed change box frontal (Mez Siemens)

- speed change proportion 1:38

- motor speed 640 /min motor input 430 W

- 4x Strain bottom from all stainless steel (blower type)

Serv. pieces :

- Complette set of padding

- Pump padding

- Staniless regulator GG DN 30 , DN 20, DN 15

- Thermo spirals 2x

- Elektroinstalation serv. pieces for each type.

Certifikation:

- Passport of used materials + certifikation

- Attest of manufecturer of operation

- Hygienic certificate of brew house

- Certificate electro

- Attest of run from manufacturer

- Documentation in english lang.

Delivery deport:

The delivery time is 10 weeks from first payment 50 % of price to account.

Duty formalitees paying the buyer.

Price ORV-50-Hi3	9860.- Eur

### Vypracoval Hapl Pavel tel: +420582345184 E-Mail pavel@haplmont.com